Hoelscher Farms – Tract 6

FIELD NOTES for a 10.08 acre tract of land out of the southwest quarter of Section 12, Block D5, EL&RR Ry. Co. Survey, Abstract 735, Lubbock County, Texas, more particularly described as follows:

BEGINNING in the west line of said Section 12 in County Road 1600 which bears N. 01º 57’ 59” E. a distance of 670.86 from a 1/2" iron rod found at the southwest corner of said Section 12 for the southwest corner of this tract.

THENCE N. 01º 57’ 59” E., along the west line of said Section 12 and said County Road 1600, a distance of 317.86 feet to the northwest corner of this tract.

THENCE S. 88º 02’ 01” E., at 30.00 feet pass a 1/2" iron rod set with a cap inscribed “OJD-ENG” (hereafter referred to as an OJD cap) in the east margin of said County Road 1600, a total distance of 1381.86 feet to an OJD cap set for the northeast corner of this tract.

THENCE S. 01º 57’ 59” W. a distance of 317.86 feet to an OJD cap set for the southeast corner of this tract.

THENCE N. 88º 02’ 01” W., at 1351.86 feet pass an OJD cap set in the east margin of said County Road 1600, a total distance of 1381.86 feet to the POINT OF BEGINNING and containing 10.08 acres of land.